TECHNICAL REPORT

Client: Botshoko Health Research Unit — Klerksdorp

Module: CMPG325 — Computer Networks

Student Name: Junia Pitso | Student Number: 39064338

Assigned IP Block: 192.168.50.0/24

1. Executive Summary

This report outlines the network infrastructure designed and deployed for the Botshoko Health Research Unit in Klerksdorp. Using Variable Length Subnet Masking (VLSM), Virtual Local Area Networks (VLANs), Router-on-a-Stick (ROAS), and automated DHCP configurations, the production network isolates departmental traffic while maximizing IP usage efficiency. Hardware logs and real-time validation test entries confirm fully operational routing paths and edge port security mechanisms.

2. Requirements and Scope

- Departmental Isolation: Separate Research, Administration, HR, IT, and Management broadcast domains.

- IP Optimization: Apply structural VLSM models to partition the 192.168.50.0/24 address space cleanly.

- Layer 3 Core Interconnectivity: Enable Inter-VLAN routing through a centralized gateway on Router R1 using subinterfaces.

- Automatic Provisioning: Establish internal dynamic lease handling using local DHCP scopes.

- Edge Access Control: Implement strict switch-level port-security restrictions on user-facing switchports.

3. Logical Addressing Plan

| Department | VLAN | Subnet | Mask | Gateway | Broadcast |
| --- | --- | --- | --- | --- | --- |
| Research | 10 | 192.168.50.0/26 | 255.255.255.192 | 192.168.50.1 | 192.168.50.63 |
| Administration | 20 | 192.168.50.64/27 | 255.255.255.224 | 192.168.50.65 | 192.168.50.95 |
| Human Resources | 30 | 192.168.50.96/28 | 255.255.255.240 | 192.168.50.97 | 192.168.50.111 |
| IT | 40 | 192.168.50.112/28 | 255.255.255.240 | 192.168.50.113 | 192.168.50.127 |
| Management | 99 | 192.168.50.128/28 | 255.255.255.240 | 192.168.50.129 | 192.168.50.143 |

Note: Subnet gateways are manually excluded from operational dynamic dynamic pools to prevent IP address conflict events.

4. Physical Topology & Switch Port Mapping

- VLAN 10 (Research): Interfaces Fa0/1 through Fa0/10.

- VLAN 20 (Administration): Interfaces Fa0/11 through Fa0/13.

- VLAN 30 (Human Resources): Interfaces Fa0/14 through Fa0/17.

- VLAN 40 (IT): Interfaces Fa0/18 through Fa0/21.

- VLAN 99 (Management): Interfaces Fa0/22 through Fa0/24.

- Backbone Infrastructure: Interface GigabitEthernet0/1 on Switch S1 operates as a secure 802.1Q boundary trunk to Router R1.

5. Router-on-a-Stick Configuration Verification

Router R1 handles inter-VLAN communications using subinterfaces G0/0.10 through G0/0.99 mapped to respective VLAN tags. Cisco IOS pool parameters successfully deliver matching configurations across all domains. Live device state verification maps dynamically to active clients:

text

R1# show ip dhcp binding

IP address       Client-ID/Hardware address   Lease expiration        Type

192.168.50.2     0001.9794.7B95               --                      Automatic

192.168.50.66    000B.BE99.AAC9               --                      Automatic

6. Switch Port-Security Configuration Audit

Switch S1 restricts interface access using dynamic sticky MAC binding. Ports will transition into an administrative shutdown state if an unauthorized machine triggers a network security violation count. Live audit status:

S1# show port-security interface fa0/1

Port Security              : Enabled

Port Status                : Secure-up

Violation Mode             : Shutdown

Maximum MAC Addresses      : 1

Total MAC Addresses        : 1

Sticky MAC Addresses       : 1

Last Source Address:Vlan   : 0001.9794.7B95:10

Security Violation Count   : 0

7. Validation Test Register

| Test Objective | Verification Command Run | Expected Functional Result | Outcome Status |
| --- | --- | --- | --- |
| VLAN Boundaries | S1# show vlan brief | Switch ports assigned onto specific VLAN domains. | PASS |
| Trunk Transport | S1# show interfaces trunk | Gi0/1 allows transport across required VLAN branches. | PASS |
| Central Gateways | R1# show ip interface brief | Logical interfaces indicate up/up configurations. | PASS |
| Routing Tables | R1# show ip route | Active routing paths visible for all 5 subnets. | PASS |
| DHCP Scope | R1# show ip dhcp pool | Client terminals receive addressing automatically. | PASS |
| Inter-VLAN Route | Simulation PDU: PC22➔PC6 | Logical path confirmation for cross-VLAN packets. | PASS |
| Edge Hardware Lock | S1# show port-security | Access ports secured against untrusted MAC paths. | PASS |
| Scaling Growth | End-node scaling addition | 8 new Research nodes register and handle traffic. | PASS |

8.Conclusion

The implemented engineering layout fully complies with the strict technical operational requirements issued for the Botshoko Health Research Unit network design. Departmental segmentation successfully mitigates rogue broadcast behaviors, and Layer 2 sticky protection preserves port integrity. Real-time validation confirms this network environment is stable, converged, and fully optimized for secure medical research applications.